Real Time Fingerprint Positioning Library (RTFPPL) for Retail Phase II

Algorithmic Design Document

Ver 1.2

Document History

| Date | Version | Comment |
| --- | --- | --- |
| 17 Mar 2016 | 1.0 | Document created |
| 8 Apr 2016 | 1.1 | RTFPPL Input Data correction |
| 11 Apr 2016 | 1.2 | Added corrections about internal frame designation. |
| 21 June 2016 |  | Added uncertainties usage in RBPF |
| 5 July 2016 |  | Added position increment uncertainties |
| 21 November 2016 |  | Updated position increment uncertainties |
| 29 May 2019 |  | RBPF for 4 biases (mag calibration on robot) |

Table of Contents

Document History	3

Table of Contents	4

1	Executive Summary	6

2	Algorithms of RTFPPL	7

2.1	Coordinate frames	7

2.1.1	Device independent frames	7

2.1.2	Fingerprint grids	10

2.1.3	Device fixed frames	11

2.1.4	Euler angles	14

2.2	The RTFPPL Input data	15

2.2.1	Initialization data – once input data	15

2.2.2	Settings – once input data	16

2.2.3	FP maps – once input data	17

2.2.4	TPN input data	17

2.2.5	RF input data	18

2.3	The RTFPPL output data	19

2.4	RTFPPL algorithm description	20

2.4.1	RTFPPL block diagram	20

2.4.2	Initialization	21

2.4.3	Coordinate converter	21

2.4.4	TPN data converter	23

2.4.5	Motion model	24

2.4.6	Wi-Fi-only position	26

2.4.7	BLE-only position	27

2.4.8	MFP-based position	27

2.4.9	Combined position	28

3	Algorithms of Uncertainties Utilization	29

3.1	Use attitude angles uncertainties	29

3.1.1	Magnetic vector uncertainty estimation	29

3.1.2	Estimation of position increment uncertainty	31

3.1.3	Algorithm of uncertainty usage in RBPF	33

3.1.4	Alternative form of RBPF	38

3.1.4.1	Alternative algorithm of RBPF	38

3.1.4.2	Sampling from likelihood	39

3.1.5	Algorithm of RBPF with 4 biases (for mag calibration on robot)	41

# Executive Summary

This report is the design documentation for the Real Time Fingerprint Positioning Library (RTFPPL) for the Retail Phase II project. It summarizes the ongoing work and the future work performed/or will be performed starting after the Proof of concept (POC) phase.

The overall architecture of the prospective library with interaction other modules is shown in picture below.

Figure 1 General diagram of Real Time Fingerprint Positioning Library

# Algorithms of RTFPPL

Algorithms of RTFPPL are designed to provide pure Wi-Fi only based position, pure BLE-only based position, MFP-based position, and combined position using data from INVN PDR.

## Coordinate frames

### Device independent frames

NED – North East Down frame with origin in device mass center.

XFFYFFZFF – Filter Frame (FF) defined as: ZFF pointed up, XFF axis belong to the horizontal plane and forms angle α with North direction, YFF completes right frame. Frame origin is defined so that all fingerprint cells have positive coordinates. The Filter Frame origin point (0, 0, 0) is located at the lowest floor among the floors, which are included in the venue map. All points at this floor in Filter Frame have Z coordinate value of zero. Note that this floor may be located at any height and it can have any number in the floor numbering system of the venue.

Note that only X and Y coordinates in Filter Frame are in meters, while Z coordinates are integer floor numbers starting at 0. However, it is convenient to use the system with traditional metric coordinates along each of the 3 axes while calculating coordinate transformations. That’s why XfYfZf is implicitly converted to such system during all the transformations and changed to its true form with integer floor number coordinates only when it is necessary.

The transformation matrix from NED to Filter Frame is:

| CNED_2_FF=cos⁡αsin⁡α0sin⁡α-cos⁡α000-1 | 1 |
| --- | --- |

Where α is the azimuth – angle between north direction and Xf axis. If the Filter Frame is rotated by α around Zf axis, then one will coincide with NED frame. Note that this rotation happens around an axis that is pointed upwards in Earth frame. D  axis of NED and  Zf axis of Filter Frame are opposite.

Another frame is used in RTFPPL – quasi-NED frame qNED. Note that this frame orientation relative to NED changes with time, therefore it is not an Earth fixed frame. At any given moment Dq axis of qNED coincides with D axis of NED and qNED evolution in time is defined by frame misalignment angle γ(t). The transformation matrix from qNED to NED is:

| CqNED_2_NED=cos⁡γ(t)sin⁡γ(t)0-sin⁡γ(t)cos⁡γ(t)0001 | 2 |
| --- | --- |

Where γ(t) is misalignment angle between NED and quasi-NED frames.

If the quasi-NED system at the time moment t is rotated around its Dq axis by angle γ(t), then the resulting rotated quasi-NED will coincide with NED.

The transformation matrix from quasi-NED to Filter Frame is defined as:

| CqNED_2_FF=CNED_2_FFCqNED_2_NED= = cosαcosγt- sinαsinγtcosαsinγt+ sinαcosγt0sinαcosγt+ cosαsinγtsinαsinγt-cosαcosγt000-1 | 3 |
| --- | --- |

Quasi Filter Frame (qFF) is used in internal calculations of particle filter. This frame orientation relative to Filter Frame changes with time, like qNED relative to NED. At any given moment ZqFF axis coincides with ZFF axis NED and qFF evolution relative to FF in time is defined by frame misalignment angle h(t). Quasi Filter Frame relates also with qNED frame by azimuth angle α that is equal to azimuth angle between Filter Frame and NED as defined in picture below.

The transformation matrix from quasi Filter Frame to Filter Frame is:

| CqFF2FF=cos⁡h(t)-sin⁡h(t)0sin⁡h(t)cos⁡h(t)0001 | 4 |
| --- | --- |

Where h(t) is misalignment angle between Filter Frame and quasi-Filter Frame.

| CqFF_2_qNED=CqNED_2_qFF=cos⁡αsin⁡α0sin⁡α-cos⁡α000-1 | 5 |
| --- | --- |

Transformation matrix from quasi-NED to Filter Frame can be also expressed by the misalignment angle ht  and the azimuth angle α as:

| CqNED_2_FF=CqFF2FFCqNED_2_qFF= = cosαcosht- sinαsinhtcosαsinht+ sinαcosht0sinαcosht+ cosαsinhtsinαsinht-cosαcosht000-1 | 6 |
| --- | --- |

According to equations (3) and (6) misalignment angles ht and γt are equal.

### Fingerprint grids

For each venue and measurement type a corresponding fingerprint database can be used by RTFPPL. The data in a fingerprint database is tied to its grid. Each grid consists of cells. The cell size can be set differently for each venue and fingerprint type.

Each fingerprint grid has a 2d array of cells for each floor in the venue map. Each cell has coordinates in the Filter Frame, which are tied to the center of the cell.

The MFP data for each cell is a 3d vector in a Magnetic Fingerprint Frame (see 2.1.1).

Note that the MFP cells still have coordinates in Filter Frame, while Magnetic Fingerprint Frame is only used to store 3 components of magnetic vector measurements in the cell. The following figure shows a MFP cell and the axis directions for each frame.

The transformation matrix from Filter Frame to MFP frame is defined as:

| CFF_2_MFP=cosπ2-sin⁡π20sin⁡π2cos⁡π20001=0-10100001 | 7 |
| --- | --- |

The transformation matrix from NED to Magnetic Fingerprint Frame is defined as:

| CNED_2_MFP=CFF_2_MFPCNED_2_FF | 8 |
| --- | --- |

### Device fixed frames

All sensor measurements are provided by OS sensor API in Sensor Coordinate System.

| Device Frame | Universal Device Fixed Frame |
| --- | --- |

XdYdZd – Sensor Coordinate System (Device Frame). Axes of the frame usually match to measurement axes of sensors. The frame defined as: origin in device mass center, Yd axis pointed to device top side, Zd axis pointed up from device screen, Xd – completes right coordinate frame. This frame is not used by TPN PDR, the transformation happens before the data reach the TPN library.

XYZ – Universal Device Fixed (UDF) frame. The frame defined as: origin in device mass center, X axis pointed to device top side, Z axis pointed down from device screen, Y – completes right coordinate frame. UDF Frame is used for all input and output of TPN PDR.

Transition from Device Frame to UDF frame is defined as the following:

| xyz=Cd2UDFxdydzd; Cd2UDF= 01010000-1; qd2UDF2=220-220 | 9 |
| --- | --- |

Hire and below in document liter C denotes transformation matrix, liter q denotes transformation quaternion.

To avoid ambiguity of heading angle, TPN PDR provides Euler angles between quasi-NED frame and one of three different Internal Device frames. Frame ID is also given. These frames are defined as described below.

- Horizontal frame. This frame is equal to Universal Device Fixed Frame.

| Universal Device Fixed Frame | Horizontal frame |
| --- | --- |

Transition matrix and frame ID defined as:

| xnynzn=C0xyz; C0=100010001; q0=1000; frame_id =0 | 10 |
| --- | --- |

- Vertical Up frame. This frame is obtained by 90 degrees clockwise rotation of Universal Device Fixed Frame around Y axis.

| Universal Device Fixed Frame | Vertical Up frame |
| --- | --- |

Transition matrix and frame ID defined as:

| xnynzn=C+1xyz; C+1=001010-100; q+1=220220; frame_id=+1 | 11 |
| --- | --- |

- Vertical Down frame. This frame is obtained by 90 degrees counter clockwise rotation of Universal Device Fixed Frame around Y axis.

| Universal Device Fixed Frame | Vertical Down frame |
| --- | --- |

Transition matrix and frame ID defined as:

| xnynzn=C-1xyz; C-1=00-1010100; q-1=220-220; frame_id =-1 | 12 |
| --- | --- |

### Euler angles

Internal Device Frames (Horizontal, Vertical Up, or Vertical Down) relates with quasi NED frame by three Euler angles:

ψ – is roll angle, counterclockwise rotation around X axis;

θ – is pitch angle, counterclockwise rotation around Y axis;

φ – is heading angle, counterclockwise rotation around Z axis.

This angles define orientation of Internal Device Frame relative to quasi-NED frame.

Rotation of Internal Device Frame (Horizontal, Vertical Up, or Vertical Down) to quasi-NED frame is performed in following order:

1-st rotation – roll around X, counter clockwise

2-nd rotation – pitch around Y, counter clockwise

3-d rotation – heading around Z, counter clockwise

Transformation matrix from Internal Device Frame to qNED frame is defined as:

| CIDF2qNED= = cosθcosψ⁡-cosφsinψ+sinφsinθcosψsinφsinψ+cosφsinθcosψcosθsinψcosφcosψ+sinφsinθsinψ-sinφcosψ+cosφsinθsinψ-sinθsinφcosθcosφcosθ | 15 |
| --- | --- |

Conversion between UDF frame and quasi NED frame can be performed by following transition matrix:

| CUDF2qNED=CIDF2qNEDCframe_id | 16 |
| --- | --- |

## The RTFPPL Input data

### Initialization data – once input data

  - Initial position and floor with uncertainties

    - Latitude, degrees

    - Longitude, degrees

    - Height from sea level, m

    - Floor

    - Latitude/Longitude covariance matrix (2x2)

    - Floor deviation – floor ambiguity

    - Floor ambiguity flag (-1: floor down only, 1: floor up only; 0 – floor up and down)

    - Height standard deviation, m

  - Initial heading with uncertainty

    - Initial heading of person, degrees – angle between N and User X axis (counter clockwise)

    - Initial heading standard deviation, degrees

### Settings – once input data

  - Venue data

    - Venue ID

    - Origin latitude φ0 – bottom left corner latitude, degrees

    - Origin longitude, λ0 – bottom left corner longitude, degrees

    - Origin altitude, m

    - Venue azimuth α degrees

    - Floor count – floor number in venue

    - X-size of venue, m – maximal available x coordinate in Filter Frame

    - Y-size of venue, m – maximal available y coordinate in Filter Frame

    - Latitude scale factor aφ

    - Longitude scale factor aλ

    - Floor height separation, m (array, TBD in future)

  - Magnetic sensor uncertainties (TBD)

  - Wi-Fi and BLE measurement uncertainties (TBD)

  - Wi-Fi/BLE/Mag/Mix position enable flags (TBD)

  - Combined position flag (MFP+Wi-Fi+BLE), (TBD)

  - MFP position flag – enables MFP position output and accounting of MFP in combined position

  - WFP position flag – enables WiFi FP position output and accounting of WiFi FP in combined position

  - BLE position flag – enables BLE FP position output and accounting of BLE FP in combined position

### FP maps – once input data

  - MFP map, (format  - TBD), now mfp3 file is used

  - Wi-Fi FP map, (format  - TBD), now wfp3 file is used

  - BLE FP map, (format  - TBD), now bea3 file is used

### TPN input data

TPN input data include motion, attitude and magnetic data. TPN data are periodical data with sample rate of 20 Hz. These data are provided by TPN PDR and include the following:

  - Time stamp, sec

  - TPN position data

    - Latitude, degrees

    - Longitude, degrees

    - Position standard deviation in north direction, m

    - Position standard deviation in east direction, m

    - User heading angle, degrees

    - Misalignment angle, degrees

    - User heading standard deviation, degrees; misalignment standard deviation proposed as: σma=α(σuhk-σuhk-1), α - empirical coefficient

    - Floor number

    - Height from sea level, m

    - Height standard deviation, m

    - Fidgeting, flag

  - TPN attitude data

    - Euler angles, i.e. roll, pitch, heading of device for conversion from Internal Device Frame to initial NED, degrees

    - Roll, pitch, heading standard deviation, degrees, provided for roll, pitch, heading of device for conversion from Internal Device Frame to initial NED

    - Internal Device Frame identifier: (0 – Horizontal frame, 1 – Vertical Up frame, -1 – Vertical Down frame)

  - TPN magnetic data

    - Uncalibrated magnetic vector in Universal Device Fixed Frame:

BTPN=BxByBzT

A dimension of each vector element is mG.

    - Covariance matrix of magnetometer bias estimation errors:

cov(bTPN)=σbx2cov(by,bx)cov(bz,bx)cov(bx,by)σby2cov(bz,by)cov(bx,bz)cov(by,bz)σbz2

Index l denotes different sample rate of these data with sample rate of magnetic vector.

A dimension of each matrix element is mG2.

    - Standard deviations of magnetometer measurements noise. A dimension is mG.

    - Magnetic data validity flag

### RF input data

- Wi-Fi observation, sample rate: 0.3-5Hz

  - Time stamp of observation

  - Wi-Fi observation vector = <Wi-Fi observation item>

Wi-Fi observation item

    - BSSID

    - RSSI, dBm

    - Central channel frequency, MHz

    - Time stamp* of measurement (optionally)

- BLE data (0.3-5Hz)

  - Time stamp of observation

  - BLE observation vector = < BLE observation item >

BLE observation item

    - UUID

    - Major

    - Minor

    - txPower  - tx Pover level for 1m distance, dBm

    - BSSID validity flag

    - BSSID

    - RSSI, dBm

    - Central channel frequency, MHz

Time tags of Wi-Fi and BLE data should be the same as of PDR data

## The RTFPPL output data

The RTFPPL calculates and outputs following data items:

- Pure BLE position

- Pure Wi-Fi position

- MFP based position

- Combined position

Each output item contains data in following format:

- Position validity

- Latitude, in degree

- Longitude, in degree

- Latitude standard deviation

- Longitude standard deviation

- Floor number

- Floor number uncertainty

## RTFPPL algorithm description

### RTFPPL block diagram

Figure shows a block diagram of RTFPPL.

Figure 2 Block Diagram of Real Time Fingerprint Positioning Library

### Initialization

The algorithm input is data according section 2.1.

The algorithm can be summarized in the following steps:

Step 1:

Step 2:

Step 3:

The algorithm output is

### Coordinate converter

Main function of Coordinate Converter is a conversion geographical coordinates to coordinates of a local 2D frame and vice versa. Additionally, CC provides conversion horizontal plane projections (2D) of vectors and covariance matrixes between the both frames.

Vectors

Local frame is defined as:

- frame origin sets in point with latitude  φ0 and longitude λ0;

- Xl axis belong to the horizontal plane and forms angle α with North direction;

- Yl axis belong to the horizontal plane and forms angle 90 degrees with Xl;

- Coefficients aφ and aλ define scaling and axis directions.

Parameters φ0, λ0, α, aφ, aλ define the conversion and have to be previously specified.

Parameters aφ, aλ are calculated by default if they are specified as zeroes:

| aφ=1Rb; aλ=-1Racos φ0 | 17 |
| --- | --- |

Where Ra and Rb are major and minor Earth semi-axes respectively.

Conversion of latitude and longitude to local frame coordinates performed as following:

| xl=cos αaφφ-φ0+sin αaλλ-λ0 yl=-sin αaφφ-φ0+cos αaλλ-λ0 | 18 |
| --- | --- |

Latitude and longitude are calculated by local frame coordinates as it is defined bellow:

| φ=φ0+aφxlcos α-ylsin α λ=λ0+aλxlsin α+ylcos α | 19 |
| --- | --- |

Vectors are converted as:

| vloc=Cgeo2locvgeo vgeo=Cgeo2locTvloc | 20 |
| --- | --- |

Where vloc=vxloc, vylocT- vector in local frame;

vgeo=vngeo, vegeoT- vector in geographical frame, vngeo, vegeo are north and east vector projection;

Cgeo2loc - is transformation matrix between geographical coordinates and local coordinates defined as:

| Cgeo2loc=cos αaφsin αaλ-sin αaφcos αaλ | 21 |
| --- | --- |

Covariance matrix transformation is performed as:

| Σloc=Cgeo2locΣgeoCgeo2locT Σgeo=Cgeo2locTΣlocCgeo2loc | 22 |
| --- | --- |

Where Σloc=σloc_xx2σloc_xy2σloc_yx2σloc_yy2 – covariance matrix in local frame;

Σgeo=σgeo_nn2σgeo_ne2σgeo_en2σgeo_ee2 – covariance matrix in geo frame, n and e indexes denote north and east components.

According to definitions above CC is used in RPFPPL for geographical coordinates and its covariance matrix conversion to coordinates of a Filter Frame and quasi Filter Frame and vice versa. Local frame represents Filter Frame or quasi Filer Frame accordingly.

### TPN data converter

TPN data converter provides conversion TPN input data to internal RTFPPL data format.

A diagram of TPN converter is given below.

Position data conversion is provided according to equations given in item 2.5.3.

Attitude data conversion consists calculation of transformation quaternions:

| qUDF2qFF=qqFF_2_qNED qIDF2qNED qframe_id qUDF2qMFP=2200-22 qUDF2qFF | 23 |
| --- | --- |

Where qqFF_2_qNED – is transformation quaternion from quasi NED to quasi Filter Frame that can be calculated from transition matrix CqFF_2_qNED  (equation  REF _Ref459907237 \h 5),

qframe_id – is transformation quaternion from UDF to current Internal Device frame, that that can be calculated from transition matrix  Cframe_id  (equations 10-12 ),

qIDF2qNED – is transformation quaternion from Internal Device frame to quasi NED, that that can be calculated from transition matrix  Cframe_id  (equation 15).

Euler angles are defined as:

| φ=φTPN θ=θTPN ψ=ψTPN+α | 24 |
| --- | --- |

Magnetic data conversion provides as follow

| B=μBTPN cov(b)=μ2cov(bTPN) σBx2σBx2σBx2=μ2σBx2σBx2σBx2TPN | 25 |
| --- | --- |

Where μ=0.1  - is relation factor between [mG] and [uT] dimensions of magnetic field representation.

### Motion model

Motion model provides updates to the position and heading of each particle. Those updates are made in the propagation function.

Motion model propagation function inputs are:

Coordinate increments estimations dXdYquasi-NED,

dFloor – floor change estimation as a floating point value.

Standard deviation of dFloor - σdFloor

Error covariance matrix for  dXdY : Σxy=σxxσxyσyxσyy

The coordinate increments dXdYquasi-NED are received in the quasi-NED frame. One of the fusion filter estimated parameters is the angle φ between quasi-NED frame and the Filter Frame. If the X axis of FF is rotated by φ around Z axis of FF, then it will coincide with North in quasi-NED.

The transformation matrix from quasi-NED frame to Filter Frame is:

CNqEqDq_2_Filter=cosφsinφ0sinφ-cosφ000-1

Noise is added to the input coordinate increments:

dXdYquasi-NED= dXdYquasi-NED+εxεy

Where εxεy is calculated based on Σxy.

Then 2x2 submatrix of CNqEqDq_2_Filter is used to convert dXdYquasi-NED to Filter Frame:

dXdYFilter=cosφsinφsinφ-cosφdXdYquasi-NED

The propagation function yields the following updates for each particle:

Pfloorupd=Pfloor+dFloorpr=Pfloor+dFloor+σdFloor*randn(0, 1)

Pxupd=Px+dXFilter

Pyupd=Py+dYFilter

Pheadingupd=Pheading+σheading*randn(0, 1)

Here randn(0, 1) is random value generated from a 1d normal distribution with 0 mean and variance equal to 1.

### Wi-Fi-only position

Position estimation is obtained from the Wi-Fi FP by comparing current Wi-Fi RSSI measurements to the data in the FP. The main problem is choosing the most probable position.

At first the likelihood function values are calculated for each cell in the fingerprint. Those values represents the likelihood of getting measurements vector z if the device was located inside that cell.

Consider an example where a Wi-Fi RSSI measurements vector 
z={z1,z2,…,zN} is received for N access points. The goal is to calculate the value of likelihood function for the FP cell xi - p(z|xi).

Two Gaussian mixture model is used to calculate the likelihood function for one RSSI measurement z.

pizxi=w1Nz;z1*,σ1+w2Nz;z2*,σ2=w11σ12πe-(z-z1*)22σ12+w21σ22πe-(z-z2*)22σ22

Here values z1*,σ1, z2*,σ2 are taken from Wi-Fi FP database.

The final value of likelihood function for the whole vector of RSSI measurements z and for cell xi is calculated as follows:

pzxi=Nj∈Npizjxi, if N>minAPcount

pzxi=0, if N≤minAPcount

Here minAPcount parameter is some constant parameter.

Then k “most probable” cells are chosen, which all have higher value of the likelihood function than the rest of the cells. Then a position estimate is calculated based on the values of likelihood function for those cells. The weight of each cell is calculated as follows:

wi=pij=1kpj,   i=1,…,k

If j=1kpj<minProbMetric, then position estimate validity flag will be set to false. Otherwise it will be set to true.

The position estimate is:

x=iwixi,   i=1,…,k

Current floor number estimate is floor number where the largest number of cells from among the k “most probable” cells are located.

### BLE-only position

BLE based positioning is provided by same way as WiFi based positioning.

### MFP-based position

(info from doxygen, TODO: correct this)

3D vector magnetic updater implementation.

The likelihood function looks as follows: pzmfpx=p(B|x) where B - is a true magnetic induction vector and x={x,y,floor,h} is a PF state vector.
We assume the following magnetometer measurement model: =Cp2l(z-b) , where z - measured vector, Cp2l  is orientation matrix and b - sensor bias.
We decompose orientation matrix as follows Cp2l=RzhCp2mroll,pitch,

Where Cp2mroll,pitch - phone to the map plane transformation matrix and Rzh - in plane rotation, which depends on heading hypothesis.
So the final Bi=Rzi(h)Cp2m(z-bi)

and assuming independency of the field components

p(zmfp|x)=p(Bxi|(x,y,floor)i)p(Byi|(x,y,floor)i)p(Bzi|(x,y,floor)i)

It is assumed that after bias compensation, each of Bx, By, Bz  components is a Gaussian distributed value with zero mean and standard deviation defined by estimated bias uncertainty.

pBix,y,floori=12πσe-(Bi-Bmapi)22σ2

### Combined position

# Algorithms of Uncertainties Utilization

## Use attitude angles uncertainties

### Magnetic vector uncertainty estimation

Definitions:

| Definition | Meaning |
| --- | --- |
| μ=μxμyμzT | MFP vector |
| φ | Roll angle (rotation around X axis) |
| θ | Pitch angle (rotation around Y axis) |
| ψ | Misalignment angle between Filter Frame and quasi-NED (rotation around Z axis) |
| σφ,θ,ψ2 | Variance of attitude angles |
| h | User heading (misalignment angle plus heading) |
| σh2 | Variance of user heading |

Assume: φ,θ, ψ  ~N(φ,θ, ψ, σφ,θ,ψ2),

Consider DCM

Q=C0C ,

where C0(φ,θ, ψ) is a non-random part of DCM rotating from MFP frame to UDF, defined as:

CMFP_2_UDF=CFIDTCIDF_2_qNEDTCqNED_2_FFTCFF_2_MFPT

matrices CFID,CIDF_2_qNED,CqNED_2_FF,CFF_2_MFP are defined in section 2.1.

C is a random part of DCM, which describes angular errors.

For small error angles (substituting sine by angle, considering cosines as unities, and ignoring terms of second order) we can write:

Q=1ψ-θ-ψ1φθ-φ1						(26)

Rotated magnetic vector from MFP frame to UDF

v=Dμ=C0CFIDTQμ=C0Cμ 		(27)

where C=CFIDTQ

Mathematical expectation of v is

Ev=C0μ								(28)

Covariance of v is

covv=Ev-Evv-EvT=C0P1C0T		(29)

Where

P1=EC-ImmTC-IT=

= σψ2μy2+σθ2μz2-σψ2μxμy-σθ2μxμz-σψ2μxμyσψ2μx2+σφ2μz2-σφ2μyμz-σθ2μxμz-σφ2μyμzσφ2μy2+σθ2μx2	,		(30)

Finally, formula (4) can be rewritten as follows:

covv=C0σψ2μy2+σθ2μz2-σψ2μxμy-σθ2μxμz-σψ2μxμyσψ2μx2+σφ2μz2-σφ2μyμz-σθ2μxμz-σφ2μyμzσφ2μy2+σθ2μx2C0T   (31)

As soon input uncertainties are given in internal frames, we transform to the UDF them according to the table ( see below):

| Angles | Uncertainties in internal frames |  |  | Uncertainties in UDF frame |
| --- | --- | --- | --- | --- |
|  | Horizontal frame | Vertical Up frame | Vertical Down frame |  |
| Roll φ | σφ2 | σψ2 | σψ2 | σφ2 |
| Pitch θ | σθ2 | σθ2 | σθ2 | σθ2 |
| Heading ψ | σψ2 | σφ2 | σφ2 | σψ2 |

### Estimation of position increment uncertainty

Position increment can be defined as follows:

∆xt=xt-xt-1=lcosν

∆yt=yt-yt-1=lsinν

where l is a distance travelled from time t-1 to t,

ν is user (or platform) heading.

We consider l and ψ as independent and normally distributed variables:

l~N(l0, σl2)

ψ~N(ψ0, σν2)

where l0 and ν0 are mean values of the distance and the user heading, σl2 and σν2 are their variances.

Position increment uncertainty, i.e. covariance of ∆X, is defined as follows:

Σ∆X=E∆X-E∆X∆X-E∆XT

where ∆X=∆x∆yT.

Due to independency of l and ν, the term (1,1) of the covariance matrix can be computed as follows:

Var∆x=Varlcosν=El2Ecos2ν-(E(l))2(E(cosν))2

Terms containing l can be easily determined as follows:

El=l0

El2=l02+σl2

Terms containing cosψ can be determined using a known integral:

-∞∞e-q2x2cos⁡sin[p(x+λ)]dx=πqe-p24q2cossin(pλ)

Using this equation, we can achieve:

Ecosν=12πσν-∞∞cosψe-ν-ν022σν2dν=e-σν22cosν0

Ecos2ν=12πσν-∞∞cos2ve-v-v022σν2dv=12+e-2σν2cos2ν0-12e-2σν2

Two forms of position increment covariance will be considered further. First, this is a case of small user heading variance σν2≪1, the second is the case of a big user heading variance σν2>1.

First of all, we consider the first case of a small user heading variance. Substituting the last expressions into formula for Var∆x and accounting that

e-α≈1-α for α≪1, we can get:

Var∆x=l02σν2sin2ν0+σl2cos2ν0

Analogously we can determine other terms of the covariance matrix. Finally, we can get the covariance matrix of position increment:

Σ∆X=l02σν2sin2ν0+σl2cos2ν0(l02σν2+σl2)sinν0cosν0(l02σν2+σl2)sinν0cosν0l02σν2cos2ν0+σl2sin2ν0

In the case or a big user heading variance, terms with negative user heading variance in the exponent became small and we can neglect them. Thus, we can get the following position increment covariance matrix, which expectedly does not depend on user heading:

Σ∆X=(l02+σl2)200(l02+σl2)2

In the particular case of user stopping, when l0=0, the last equation is transformed to the following covariance matrix:

Σ∆X=σl2200σl22

Thus, it is possible to adapt the position increment covariance matrix based on the following input variables:

- user heading standard deviation σν,

- user (or platform) heading ν0,

- indicator of user stopping.

### Algorithm of uncertainty usage in RBPF

The state vector consists of two parts

Xt=XtlXtn

where the Xtl denotes the state variable with linear dynamics and Xtn denotes nonlinear state variable.

The former state variable contains three biases of a magnetometer:

Xtl=bxtbytbztT

The latter state variable contains coordinates of a user:

Xtn=xtytψtftT

where x and y are Cartesian coordinates, ψ is a misalignment angle between Filter Frame and quasi-NED, and f is a floor.

At time t, the m-th particle, m=1, …, M, M is the number of particles, is defined as XtnXtlPtm, where Pt is a covariance of Xtl.

At time t, the input variables are as follows:

∆xt and ∆yt are increments of Cartesian coordinates (in fingerprint frame),

σEt and σNt are position standard deviation in NED (ENU) frame,

Btk=BxtkBytkBztkT are raw magnetometer readings, k=1,…,K; K is the number of readings between two consecutive moments of time, t-1 and t,

φtk,θtk, ψtk are roll, pitch, and heading correspondingly,

σψtk, σθtk, and σφtk are standard deviation of heading, pitch and roll correspondingly,

σm2 is a variance of magnetometer measurement noise (assumed to be equal for each axis),

M=μij,Diji=1,…,I;  j=1,…,J is the I×J×3 MFP grid; every cell contains a mean vector μij=μxijμyijμzijT and a covariance matrix of normal distribution. The covariance matrix is diagonal:

Dij=dxij2000dyij2000dzij2

The state model (in BF) can be defined by the following equations:

Xtn~pXtnXt-1n,

Xtl=Xt-1l+ξt,

zt= CMFP2UDFMXtn+Xtl+ηt+ζt,

Where:

ξt~N0,σb2I3×3 is a noise of magnetometer bias estimate,

ηt~ N0,CMFP2UDFD(Xtn)CMFP2UDFT is a noise of a magnetic fingerprint map,

ζt~N0,σm2I3×3 is noise of a magnetometer,

CMFP2UDF is DCM rotating from MFP to UDF.

The algorithm of RBPF is as follows (without floor estimation):

- Compute position uncertainties in the Filter Frame (FF):

ΣΔXFF=σxt2……σyt2=CNED_2_FPFΣΔXNEDCNED_2_FPFT

where:

ΣΔXNED is the position increment covariance in the NED frame (see the previous section),

CNED_2_FPF=cos(αt)sin(αt)sin(αt)-cos(αt).

The distance l02 can be either computed as travelled from time t-1 to t or set as averaged step length. The distance variance σl2 can be set as a constant.

- Prediction of Xtn in PF:

xtm=xt-1m+∆xtcosψtm-∆ytsinψtm+nxtm,

ytm=yt-1m+∆xtsinψtm+∆ytcosψtm+nytm,

ψtm=ψt-1m+nψtm,

nxtmnytm~N00,ΣΔXFF,

nψtm~N0,σψt2,

m=1, …, M.

- Prediction of Xtl in KF:

btm=bxtmbytmbztmT,

bt,t-1m=bt-1,t-1m (predicted linear state),

Pt,t-1m=Pt-1,t-1m+σb2I3×3 (predicted state covariance),

Stkm=Pt,t-1m+Qtkm (predicted measurement covariance),

where the covariance matrix Qtmis defined as follows:

Qtkm=CMFP2UDFkσψ2μyij2+σθ2μzij2+dxij2-σψ2μxijμyij-σθ2μxijμzij-σψ2μxijμyijσψ2μxij2+σφ2μzij2+dyij2-σφ2μyijμzij-σθ2μxijμzij-σφ2μyijμzijσφ2μyij2+σθ2μxij2+dzij2(CMFP2UDFk)T++σm2I3×3

zt,t-1mk=bt,t-1m+CMFP2UDFmkμ(xtm,ytm) (predicted measurement),

where μxtm,ytm=μxmμymμzm is a fingerprint of a cell corresponding to coordinates of m-th particle,

m=1, …, M.

- Update of Xtn in PF:

wtm=wt-1m∙1(2π)3Stkme-12Btk-zt,t-1mkTStkm-1Btk-zt,t-1mk, m=1,…,M,

where averaging is applied for all k=1,…,K.

A simplification is possible if we assume that the predicted measurement covariance does not depend on k, i.e. Stkm≈Stm. Then

Btk-zt,t-1mkTStm-1Btk-zt,t-1mk=Btk-zt,t-1mkTStm-1Btk-zt,t-1mk+TrStm-1Σ

where Σ is covariance of Btk-zt,t-1mk.

Neglecting the last term, we have the following equation:

wtm=wt-1m∙1(2π)3Stme-12Btk-zt,t-1mkTStm-1Btk-zt,t-1mk, m=1,…,M,

where Bt and zt,t-1m are averaged of Btk,  zt,t-1mk, k=1,…,K.

- Normalize weights wtm, m=1,…,M.

- Resample XtnXtlPtm, m=1,   , M.

- Update of Xtl in KF:

Kt=Pt,t-1mStm-1,

bt,tm=bt,t-1m+KtBt-zt,t-1m,

Pt,tm=Pt,t-1m-Pt,t-1mStm-1Pt,t-1m.

- Output positon and uncertainty estimate:

ZtFPF=m=1Mwtm-1m=1MwtmZtm

ZtNED=CFPF_2_NEDZtFPF

ΣZ=CFPF_2_NEDm=1Mwtm-1m=1Mwtm(Ztm-ZtFPF)(Ztm-ZtFPF)TCFPF_2_NEDT

where Ztm=xtmytm.

- Simplification of averaging in step 4 is possible if matrix CMFP2UDFmk is rewritten in the following form:

CMFP2UDF=mkCFIDTCIDF_2_qNEDTCqNED_2_FFTCFF_2_MFPT=CFIDTCIDF_2_qNEDTCqNED_2_qFFTCqFF2FFTCFF_2_MFPT

Due to a special form of matrix CFF_2_MFPT, we can swap two last matrixes:

CMFP2UDFmk=CFIDTCIDF_2_qNEDTCqNED_2_qFFTCFF_2_MFPTCqFF2FFT

In the right part, only the last matrix depends on index of a particle m, whereas four preceding matrixes depend on index of readings k. Let us insert the following depictions:

C1k=CFIDTCIDF_2_qNEDTCqNED_2_qFFTCFF_2_MFPT

C2m=CqFF2FFT

As a result, we achieve:

CMFP2UDFmk=C1kC2m

Now we can rewrite predicted measurement as follows:

zt,t-1mk=bt,t-1m+C1kC2mμ(xtm,ytm)

In the last equation, only matrix C1k depends on k. This allows simplifying averaging by k (see step 4):

Btk-zt,t-1mk=Btk-bt,t-1m-C1kC2mμxtm,ytm

Thus, we only need to average by k magnetic measurements Btk  and the matrix C1k. These operations can be realized in advance, before call of the particle filter.

- Adaptive RBPF serves for estimating the measurement error matrix R. Now a particle is defined as follows: XtnXtlPtRtm.

Innovations are used for adaptation:

vtm=Bt-zt,t-1m=Bt-bt,t-1m-C1kC2mμxtm,ytm

Covariance matrix of innovations includes covariance of the bias noise,  covariance of the measurement noise, and MFP map uncertainty:

EvtmvtmT=E(Bt-bt,t-1m-C1kC2mμxtm,ytm)Bt-bt,t-1m-C1kC2mμxtm,ytmT=Pt,t-1m+Rtm+C1kC2mdxij2000dyij2000dzij2C2mTC1kT

Estimate of measurement error covariance matrix can be derived from the previous equation:

Rtm=1Ll=0L-1vt-lmvt-lmT-Pt,t-1m-C1kC2mdxij2000dyij2000dzij2C2mTC1kT

where L is the length of moving window for estimation of the innovation covariance matrix.

To prevent obtaining non-positive definite matrix Rtmdue to the matrix subtraction, minimal values of diagonal elements of Rtm must be limited by some positive values.

### Alternative form of RBPF

### Alternative algorithm of RBPF

An alternative algorithm of RBPF is as follows:

For m = 1:M

Xtm=MotionModel(Xt-1m,ut)

wtm=MeasurementModel(Bt,Xtm,M)

End

For m = 1:M

r~U0,1

If r≤1-α

Xtm=SamplingWith ReplacementXt,w

Else

Xtm=SamplingFromLikelihoodBt,Xt,M

End

wtm=1M

End

In this algorithm, definitions are like in previous sections with the following exceptions:

α is a part of particles that are generated from likelihood (mixing ratio),

ut=∆xt∆yt…T  .

This algorithm generates 1-αM particles traditionally from a motion model whereas αM particles are generated alternatively, from likelihood. α can be 0.05-0.1.

### Sampling from likelihood

The most difficult part of the alternative algorithm is sampling from likelihood. The likelihood can be written as follows:

LtmBtψtm,M(xtm,ytm)=12π3Dtmexp-12Bt-btm-Ct ψtm,φt,θtμxtm,ytmTDtm-1Bt-btm-Ct ψtm,φt,θtμxtm,ytm

where Dtm=D(xtm,ytm)

Ct ψtm,φt,θt=C2(ψtm)C1(φt,θt)

We use a simplified sampling procedure:

- Set an area around weighted average of xtm,ytm, say 10 by 10 cells of MFP map, and number every cell of the area, q=1,…,qmax

- Find heading for every cell:

ψtq=atan(Btx-btx)μyxq,yq-(Bty-bty)μxxq,yq(Btx-btx)μxxq,yq+(Bty-bty)μyxq,yq

where xq,yq are coordinates of the center of the cell q,

μ=C1(φt,θt) μ ,

bt is a weighted average bias of a magnetometer.

- Calculate likelihood for every cell, q=1,…,qmax

LtqBtψtq,M(xq,yq)=12π3Dtqexp-12Bt-bt-Ct ψtq,φt,θtμxq,yqTDtq-1Bt-bt-Ct ψtq,φt,θtμxq,yq

where Dtq=D(xq,yq)

- Normalize set of Ltq, q=1,…,qmax :

vtq=Ltqq=1qmaxLtq

- Generate αM particles using normalized weights vtq. Particle m sampled with weight vtq has the following components:

xtm=xtq,  ytm=ytq,  ψtm=ψtq,  btm=bt

Further modifications of the algorithms can be as follows:

- Blur of xtm and ytm using uniform distribution with spread of cell size; blur ψtm using normal distribution with ψtq as a mean value; variance of ψtm will be tuned by simulation.

- For economy of computations, execute the second part of the algorithm (sampling) once per several calls.

### Algorithm of RBPF with 4 biases (for mag calibration on robot)

This section describes difference from a basic RBPF algorithm of section 3.1.3.

The state vector consists of two parts

Xt=XtlXtn

where the Xtl denotes the state variable with linear dynamics and Xtn denotes nonlinear state variable.

The linear state variable contains three biases of a magnetometer and one bias of Z component of MFP DB:

Xtl=bxtbytbztbFtT

The biases of a magnetometer  bxtbytbzt are in device frame whereas the bias of Z component bFt of MFP is in FP frame.

- Prediction of Xtl in KF:

btm=bxtmbytmbztmbFtmT,

bt,t-1m=bt-1,t-1m (predicted linear state),

Pt,t-1m=Pt-1,t-1m+σb20000σb20000σb20000σF2 (predicted state covariance, 4x4),

Stkm=HPt,t-1mHT+Qtkm (predicted measurement covariance),

where H=100001000010 (3x4 matrix)

We assume that the predicted measurement covariance does not depend on k, i.e. Stkm≈Stm.

zt,t-1mk=Hbt,t-1m+CMFP2UDFmkμxtm,ytm-00bFtm (predicted measurement),

Applying averaging as was described before, we get the following equation:

zt,t-1mk=Hbt,t-1m-C1kC2mμxtm,ytm-00bFtm

- Update of Xtn in PF:

wtm=wt-1m∙1(2π)3Stme-12Btk-zt,t-1mkTStm-1Btk-zt,t-1mk, m=1,…,M,

where Bt and zt,t-1m are averaged magnetometer reading Btk and averaged  zt,t-1mk, k=1,…,K.

- Update of Xtl in KF:

Kt=Pt,t-1mHStm-1, (KF gain, 4x3 matrix)

bt,tm=bt,t-1m+KtBt-zt,t-1m,

Pt,tm=Pt,t-1m-Pt,t-1mHTStm-1HPt,t-1m.